# Audio Flash Memory Upgrade

The VP-730/731 audio firmware is located in FLASH memory, which lets you upgrade to the latest Kramer firmware version in minutes! The process involves:

- Downloading from the Internet (see section 1.1)

- Connecting the PC to the AUDIO PROG. terminal block connector (see section 1.2)

Important Note: in early batches of VP-730/731 the AUDIO PROG RS-232 connector and the AUDIO PROG button are located inside the device. Accessing them must be done only by qualified technician. Please contact your local Kramer office for assistance.

- Upgrading Firmware (see section 1.3)

## Downloading from the Internet

You can download the up-to-date file from the Internet. To do so:

1.	Go to our Web site at  www.kramerelectronics.com and download the file: “FLIP_VP730-731.zip” from the Technical Support section.

2.	Extract the file: “FLIP_VP730-731.zip” to a folder (for example, C:\Program Files\Kramer Flash).

3.	Create a shortcut on your desktop to the file: “FLIP.EXE”.

## Connecting the PC to the RS-232 Port

Before installing the latest Kramer audio firmware version on a VP-730/731 unit, do the following:

1.	Connect the PC to the AUDIO PROG. terminal block connector, as defined in Figure 1 and Table 1:

Figure 1: RS-232 PINOUT Connection

Table 1: RS-232 PINOUT Connection

| Connect this PIN on the Terminal Block Connector: | To this PIN on the 9-pin D-sub Connector |
| --- | --- |
| Tx | PIN 2 |
| Rx | PIN 3 |
| GND | PIN 5 |

2.	Push the AUDIO PROG. button using a small screwdriver.

3.	Switch the unit ON.

Note: this sequence is critical – first push the AUDIO PROG button and then turn on the unit

## Upgrading the Audio Firmware

Follow these steps to upgrade the audio firmware:

1.	Double click the desktop icon: “Shortcut to FLIP.EXE”.
The Splash screen appears as follows:

Figure 2: Splash Screen

2.	After a few seconds, the Splash screen is replaced by the “Atmel – Flip” window:

Figure 3: Atmel – Flip Window

3.	Press the keyboard shortcut key F2 (or select the “Select” command from the Device menu, or press the integrated circuit icon in the upper right corner of the window).
The “Device Selection” window appears:

Figure 4: Device Selection Window

4.	Click the button next to the name of the device and select from the list: AT89C51RD2:

Figure 5: Selecting the Device Window

5.	Click OK and select “Load Hex” from the File menu.

Figure 6: Loading the Hex

6.	The Open File window opens. Select the correct HEX file that contains the updated version of the firmware for VP-730/731 (for example 44M_V1p2.hex) and click Open.

7.	Press the keyboard shortcut key F3 (or select the “Communication / RS232” command from the Settings menu, or press the keys: Alt SCR).
The “RS232” window appears. Change the COM port according to the configuration of your computer and select the 9600 baud rate:

Figure 7: RS-232 Window

8.	Click Connect.
In the “Atmel – Flip” window, in the Operations Flow column, the Run button is active, and the name of the chip appears as the name of the third column: AT89C51RD2.
Verify that in the Buffer Information column, the “HEX File: VP730.hex” appears.

Figure 8: Atmel – Flip Window (Connected)

9.	Click Run.
After each stage of the operation is completed, the check-box for that stage becomes colored green.
When the operation is completed, all 4 check-boxes will be colored green and the status bar message: Memory Verify Pass appears:

Figure 9: Atmel – Flip Window (Operation Completed)

10.	Close the “Atmel – Flip” window.

11.	Disconnect the power on the VP-730/731.

12.	If required, disconnect the rear panel AUDIO PROG. terminal block connector on the VP-730/731.

13.	Release the rear panel AUDIO PROG. button, using a small screwdriver.

14.	Connect the power to the VP-730/731.
Upon initialization, the new VP-730/731 audio software version shows in the Information menu.

Upgrade should be carried out by skilled technical personnel. Failure to upgrade correctly will result in the malfunction of the machine

The files indicated in this section are given as an example only. File names are liable to change from time to time

See also the blue progress indicator on the status bar

If an error message: “Not Finished” shows, click Run again